Solvent optimization in the Recrystallization

purification of PharmacEUtical Products

Richard J. Fickelscherer, PE,   Stephanie A. Morrissey   &  Christian M. Ferger

DEPARTMENT   OF   CHEMICAL   &   BIOLOGICAL   ENGINEERING

STATE UNIVERSITY OF NEW YORK AT BUFFALO,  Buffalo, NY, USA   14260

Abstract:   This treatment describes the details of a methodical three step algorithm for determining the optimal operating conditions for the recrystallization separations of solid mixtures.  Our algorithm was applied to optimally separate a representative pharmaceutical product (Caffeine) from a related pharmaceutical product (Theophylline).  The limitations of such calculations with currently available, widely used predictive methods for computing solution thermodynamics without experimental data are directly examined.  Also presented here is a novel two stage recrystallization procedure which can potentially dramatically improve the recovery yields of the desired products.  The systematic optimization calculations described herein should enable researchers to quickly screen many potential solvent systems and operating conditions and concentrate experimental efforts only on the most promising candidates for such purifications.

# Keywords:   recrystallization optimization, mutual solubility calculation, pharmaceutical product purification, optimal solvent system selection, counter-recrystallization

# 1.0   Introduction

# In the pharmaceutical industry, many desired solid intermediate and product compounds are purified with recrystallization separations.  Such separations are chosen typically due to: 1) thermal stability issues of the products at the excessive temperatures required to perform other potential separation techniques; 2) advantages of retrieving pure solid compounds directly; and/or 3) recovery yield advantages when compared to alternative separation techniques.  The biggest disadvantage of recrystallization purifications is that complete recovery of the desired solid product is not possible, as some of it is left in the raffinate solution.  Even small potential losses here can represent substantial revenue hits for highly valuable pharmaceutical intermediates or products.  Consequently, maximizing the recrystallization recovery yield is often the most sought-after goal of the researchers designing such separations.

# Recovery yield of the desired product is dependent upon the specific solvents and operating conditions chosen to be utilized for performing these separations.  Recovery yield will be shown to be directly dependent upon the saturated solubilities of the given solutes being separated in those solvents and at those operating conditions.  Hence, choosing the optimal solvent/solvent mixture and operating conditions for maximizing these recovery yields thus requires both a convenient and highly accurate means to determine the associated solutes’ saturated solubilities.  Directly measuring them in all potential solvent/solvent mixtures and operating conditions of interest is normally both too highly expensive and time consuming to be considered practical.  Consequently, to help rectify this situation, computational methods are presented in this treatment which greatly narrow down the required search to only the most promising solvent/solvent mixtures and operating conditions.  These results can then be experimentally verified to be the most optimal in recovery yield of the desired products.  This optimization algorithm therefore helps to focus that experimental search to only the most promising solvent/solvent mixture candidates and operating conditions for use in the recrystallization purification.  It should allow the best solvent system to be specified very early in the process system development, long before it is locked into the IQ/OQ protocol.

# General Problem of Solvent Optimization

# The procedures used in optimization are very general and thus have wide ranges of application.  The quantity to be optimized is first mathematically described by the independent variables that affect it.  This description is known as the objective function.  The independent variables are subject to constraints that represent the limits of physical reality or consider other externally imposed operating barriers.  These constraints directly limit the search space over which the optimization procedure is performed and help ensure that the results of the optimization are meaningful.  Optimization occurs when a set of specific values of these independent variables is found which gives the desired extreme of the objective function.

In solvent optimization, the recovery yield of the desired purified product is the quantity to be maximized.  This yield must therefore first be mathematically described as a function of the adjustable system variables.  These variables include the choice of solvent compounds and their concentrations in the resulting solvent mixture, the amount of solvent mixture required, and the operating temperatures and pressures.  The following analysis is based on thermodynamic equilibrium limits and is not concerned with the complex problems relating to potential polymorph species, nucleation, crystal seedings and the rates of crystal growth, the crystal size distribution, final crystal washing or the downstream processing of the raffinate solution.  Solvent systems and their associated operating conditions are thus subsequently ranked according to the corresponding calculated yields of the desired products: the highest yield will indicate which solvent system and operating conditions deserve further consideration for the actual recrystallization separation.

# 2.1   Objective Function for Maximizing Recovery Yield

The following thought experiment can be used to formulate the desired objective function.  Consider a solid mixture containing two compounds, where Compound 1 is the desired product and Compound 2 is the undesired species to be removed.  In this solid mixture, the two compounds have mole fractions s1 and s2, which satisfy the usual condition:

- 1.0   =   s1   +   s2

The rest of this analysis will be based on one mole of this solid mixture at constant temperature and pressure.

Starting with a set solvent mixture, small amounts of it could be added to this solid mixture and left to equilibrate.  Initially, the liquid phase would be saturated with both solid compounds.  The remaining solid mixture would be undergoing composition changes, but the liquid phase composition would be constant as the solvent mixture continued to be added.

A point would eventually occur where just enough of the solvent mixture had been added to cause one of the solid mixture compounds to be totally dissolved.  For discussion purposes, assume that the remaining solid compound is Compound 1.  If the solvent mixture continued to be added after this point, the remaining pure solid phase would eventually disappear.  Thus, the maximum recovery yield of pure Compound 1 occurs at the last point of solvent addition in which Compound 2 is still saturated in the liquid phase.  For the given system temperature and pressure, the amount of the solvent mixture added to reach that point is considered the optimal amount.  If slightly less than the optimal amount was added, the yield of Compound 1 would be higher, but the recovered solid would not be pure.  If slightly more was added, the recovered compound would still be pure Compound 1, but the yield would be lower.  Therefore, the optimal stopping point for a recrystallization would correspond to the solution composition which would result from the addition of the amount of the solvent mixture which just dissolves all of Compound 2.  This point corresponds to the local maximum recovery yield using this solvent mixture at this temperature and pressure.

The above discussion can be formulated by considering the equilibrium conditions of the saturated solutions and applying mass balances.  The minimum amount of solvent required to cause Compound 2 to just become undersaturated in solution is represented by:

- L	=   (s2 / x2**) * [1.0 – (x1** + x2**)]

At this point, the fractional amount of Compound 1 present in the initial solid mixture that is now in solution is represented by:

- N   =   (x1** / s1) * (s2 / x2**)

The ratio of x1** / x2** is defined as the Solvent Selectivity factor (Sij) between Compounds 1 and 2 for that solvent mixture and operating temperature and pressure, thus:

- S12   =   x1** / x2**

The Solvent Selectivity factor is a direct indication of the efficiency of those conditions for conducting the desired recrystallization.  Values lower than 1.0 are sought to maximize the recovery yields of the desired product possible with recrystallization purifications.  This is akin to the measure of relative volatility of compounds undergoing distillation separation.

For the optimal solvent loading indicated, the recovery yield of pure Compound 1 left in the solid phase is thus:

- Y   =   1.0   -   N   =   1.0   -   (x1** / s1) * (s2 / x2**)   =   1.0   -   S12 * (s2 / s1)

This is the desired objective function that needs to be maximized through the procedure of solvent optimization.  Other researchers [3] have used different objective functions in their solvent screenings for optimizing recrystallizations.

2.2	Three Step Algorithm in Solvent Optimization

Solvent optimization proceeds by the following three step algorithm:

For a fixed composition of solvents in a solvent mixture, the only independent variables affecting Equation 5 are the system temperature, pressure, and the amount of solvent loaded.  For pressures much below the critical pressure of the solution, it is usually a good approximation to assume the liquid phase is incompressible [4], thus not a function of pressure [2].  At a fixed temperature, it has been shown that there is an optimal amount for the solvent mixture loaded.  As the temperature is changed, the optimal amount of solvent mixture loading and the local maximum yield both change.

Once the regional optimum temperature has been found, the next step in the global optimization would be to adjust the concentrations of the individual solvents in the solvent mixture.  The previous search would then be performed again.  The result would be the maximum recovery yield possible with the optimal mixture of those specific solvents at the corresponding optimal temperature.

The final step in the solvent optimization algorithm would be to change the members of the solvent mixture.  The two previous searches would be repeated until the global optimum recovery yield was determined.  This would be the thermodynamic maximum yield possible for the recrystallization separation of the solid mixture from a single recrystallization.  However, these last two steps represent the most arduous of the three steps in this algorithm because of the enormous number of potential solvents and even immensely greater number of possible solvent mixture combinations.

Fortunately, the search space of choosing potential candidate solvents can be constrained by factors not taken into account in the derivation of the objective function.  These include such solvent characteristics as toxicity, reactivity with the other solution components, corrosiveness, phase behavior at the operating conditions, flammability, availability, and cost.  Such considerations have been used in other solvent optimization studies [3] to identify a list of 62 solvents most commonly used in research centers and industries.  Nonetheless, even with this dramatic reduction in potential candidate solvent mixture choices, an enormous resulting search space for conducting an exhaustive solvent optimization such as that described above still exists.  A simplification to this exhaustive search routine is consequently described later in order to make this search more efficient.

The three steps of this systematic solvent optimization algorithm represent a simple but methodical procedure for maximizing the recovery yield of the desired product.  This algorithm directly considers the limits imposed by the phase equilibria of solid compounds saturated in liquid solutions.  At its core, the effects that the solvent choices and operating conditions have upon this algorithm’s objective function are implicitly buried in these saturated solution mole fractions.  Therefore, the prediction of the solid compounds’ saturated solubilities as a function of the various independent variables is the paramount calculation in solvent optimization.

3.0	Solubility Predictions

A common feature of all separation processes is the exploitation of some difference in exhibited behavior of the compounds being separated when placed in an identical environment.  In recrystallization separations conducted at relatively low pressures, the temperature and/or chemical composition of the mother liquor can be manipulated to cause the selective formation of the desired solid phase.  The undesired compounds remain in the raffinate solution.  The separation thus occurs because of the limited mutual saturated solubilities of the various solutes.  Consequently, highly accurate predictions of the saturated solubilities of solution compounds are essential for correctly determining the optimal operating conditions for these purifications.  Saturated solubilities of single solutes in solution can be directly predicted by considering the underlying solid-liquid phase equilibrium conditions.  The following standard equation is used to calculate the saturated solubility of a single solute in a given liquid.  Its derivation is described in detail by Sandler [5]:

- ai*   =   exp[(- ΔHiFUS / R*T)   *   ((TMi   -   T)   /   TMi)]

Equation 6 represents a very useful means for predicting saturated solubilities of non-solvated, non-electrolytic compounds in a wide range of liquid solutions.  In our studies, the UNIFAC Equation [6] was used to determine the activity coefficient of Compound i in the above equation.  This equation’s use to demonstrate solvent optimization was chosen in our study solely because it does not require any additional experimental data to determine those coefficients.  Its use has been previously shown to typically give good (but at worst only ball park) estimates of actual activity coefficients if the following conditions identified by Wilcox [7] are adhered to: 1) the components in the liquid solution should all be well below their critical points; 2) the system pressure should not be greater than a few atmospheres; 3) the system temperature should be in the range of 300K to 425K; 4) the solution cannot contain electrolytes or non-condensables; 5) the solution should not contain polymer molecules; and 6) each solution molecule should contain ten or fewer UNIFAC functional group types.

Computational methods for estimating saturated solubilities are usually necessary because saturated solubility data recorded in the open literature are fairly scant.  Usually they are reported for a given pure solid saturated in a few pure solvents at only a few temperatures.  Furthermore, many times saturated solubilities are unfortunately specified in qualitative terms such as slightly soluble or moderately soluble.  For the vast majority of the exotic compounds of interest to the pharmaceutical industry, numeric saturated solubility data just do not currently exist in the open literature.  Furthermore, for most of those compounds for which such data do exist vary significantly at a given temperature due to polymorphs of those solids.

As previously discussed, accurate predictions of saturated solubilities are essential in order to optimize the operating conditions of recrystallization purifications.  Equation 6 represents a straightforward method for predicting such solubilities.  It requires the values of normal melting points and heats of fusion for each solid being separated.  Since these values can be measured directly from pure compounds, the experiments required to obtain them are relatively easy to perform.  Equation 6 also requires a method for calculating the activity coefficient of the saturated compound.  Also as was previously discussed, this can be directly done with the UNIFAC Equation, which requires that the values of surface areas, volumes, and interaction parameters of all the required functional groups be available.  This is generally true for all of the normally encountered functional groups.  The utility of Equation 6 combined with the UNIFAC equation has been repeatedly demonstrated to be an effective means of calculating the solubilities of solitary, but chemically simpler, solids in pure liquid solvents [17-20].  For more chemically complicated pharmaceutical compounds, use of the UNIFAC equation has proven less successful at accurately computing activity coefficients [21], although it still generally gives correct trends in them.  Consequently, without other experimental data, those poor estimates of these coefficients are the best that are easily available.

- Computational Studies and Results

Most chemical reactions of interest in pharmaceutical manufacturing often lead to side products which, although they differ by only one or so functional groups from the desired compound, are still quite chemically and physically similar to that compound.  These side products must typically be separated out from the desired reaction product either before further reactions proceed or in order to purify the desired product.  The chemical similarity of these side products to the desired product makes this a difficult task.  Experience and ingenuity are thus required by the researchers to efficiently handle these separations so as to allow the highest profitable outcome.  As discussed, this equates to maximizing the recovery yield of the desired reaction product when utilizing recrystallization purification.

In this study, we chose the target desired pharmaceutical product to be Caffeine. It is to be purified from solid mixtures containing a chemically similar product, Theophylline.  Caffeine is the most highly consumed psychoactive substance in the world [22].  Theophylline anhydrous is widely used as a bronchodilator for the treatment of chronic asthma [24].  The physical and chemical properties of these compounds are listed in Appendix A in Table A.1 and Table A.2, respectively. Throughout the following discussion, assume we are trying to purify a solid mixture which is 85 mole percent Caffeine (i.e., s1 = 0.85) and 15 mole percent Theophylline (i.e., s2 = 0.15).

The solvents used in this study were chosen chiefly because all relevant required UNIFAC parameters for our target pharmaceutical products exist in the open literature.  The twenty representative solvents chosen for this study and their relevant chemical and physical properties are listed in Table A.3.   The largest calculated yields of pure caffeine possible with each potential pure solvent are listed in Table A.4.  Where available, comparisons of literature single compound solubilities to calculated values using the UNIFAC Equation are given in Table A.5.

5.0	Discussions

As is obvious from Table A.5, the calculated solubilities using the UNIFAC Equation range from good to fair to mostly poor estimates of the corresponding measured values.  This result is not unexpected for pharmaceutical products [21].   It should also be noted that only a few measured solubilities exist in the literature for these products, in 13 target solvents for Caffeine and in only 4 target solvents for Theophylline.  Without the availability of actual, unlimited solubility data, the need to use less accurate methods to estimate these solubilities is the only avenue left available to directly perform solvent optimization calculations.  It must be stressed again that the optimization algorithm presented here can use any available activity coefficient estimation method besides the UNIFAC Equation if the requisite experimental data is available.  Again, the UNIFAC Equation was used here merely because of the typical absence of this required data for the pharmaceutical products of interest.

As previously discussed, the Solvent Selectivity factor given by Equation 4 is a direct measure of the recrystallization efficiency.  Efficient recrystallizations require that the undesired compound has a relatively high solubility as compared to the solubility of the desired compound; that is, the Solvent Selectivity factor should be much less than 1.0.  However, the computed minimum solvent selectivity factors for Caffeine and Theophylline in the twenty various studied pure solvents at 50oC are all between 2.92 and 6.11.

Since the Solvent Selectivity factor is a direct result of the thermodynamic phase equilibrium, methods to force it lower for a given solvent system and operating temperature are impossible.  Consequently, rather than trying to struggle against actual physical barriers, it is more appropriate to try to take advantage of such relatively high Solvent Selectivity factors.  Forthwith, if the actual factor for a given solvent/solvent mixture is high enough, it is possible to dramatically increase the recovery yield of Caffeine by performing two crystallizations in series.  This can occur only if one of the target solvent mixtures has a Solvent Selectivity factor S12 greater than 5.67 (= 0.85 / 0.15).  This is the minimum value of this factor required to cause the impurity (Theophylline) to recrystallize out of the solution before the desired product (Caffeine).  For such a solvent mixture, it would be possible to improve the overall recovery yield by performing the following clever sequence of separation processes.

Using a suitable solvent mixture having a Solvent Selectivity factor greater than the 5.67 minimum threshold, a recrystallization could be performed.  The result would be the formation of a solid phase containing only Theophylline: all the Caffeine in the original solid mixture would be left in the raffinate solution, as would some of the initial Theophylline.  This procedure can be thought of as “counter recrystallization,” [21] because it purifies the impurity rather than the desired compound.  It is still however useful nonetheless because a separation in the direction of eliminating the impurity occurs.  A second standard recrystallization would then be conducted on the solid residue of the resulting evaporated raffinate solution with the solvent corresponding to the lowest Solvent Selectivity factor for the desired purified product.  This sequence of separations consequently represents a novel means to maximize possible recovery yield from the recrystallization purification of pharmaceutical products.

To demonstrate the potential improvement in overall recovery yield with “counter recrystallization” followed by normal recrystallization, calculations were performed exhaustively using all 190 possible binary combinations of the 20 solvents listed in Table A.4.  The results are listed in Table A.6. From Table A.6, it is apparent that a dramatic improvement in overall recovery yield is possible when the indicated “counter recrystallizations” are also performed.  Using two such recrystallizations in series is the recommended method for purifying Caffeine from Theophylline in such 85%/15% mole percent Caffeine/Theophylline mixtures.

Since effective “counter recrystallization” separations are only possible when the Solvent Selectivity factor is above the required threshold, it is useful to examine the consequences resulting from changing the value of this threshold.  For an arbitrary binary solid mixture containing 30 molar percent impurity, the minimum threshold Solvent Selectivity factor limit would be 2.33 (= 0.70 / 0.30).  However, for a relatively highly pure mixture of the same compounds containing only 5 molar percent impurity, this minimum threshold increases to 19.0 (= 0.95 / 0.05).  This higher value of the minimum threshold would eliminate many of the candidate solvents which would be considered acceptable for “counter recrystallization” of the more impure mixture.  Consequently, “counter recrystallization” would generally be a useful procedure for the initial cut at such purifications only for moderately and highly impure initial solid mixtures.

Lastly, it is also recommended that the search for optimal solvent systems be first conducted only with pure solvents, not mixtures of solvents.  In general, such mixtures did not tend to either dramatically decrease or increase the Solvent Selectivity factors as was initially expected: the optimal yields almost always occurred at one of the pure solvent extremes of the given solvent mixtures and also at one of the two possible extreme operating temperatures.  Performing the exhaustive search on pairs of binary mixtures as was conducted in this study is O(N2) computationally complex, compared with O(N) complexity for using only pure solvents (N is the number of target pure solvents under consideration).  This thus represents a great deal of wasted search to find the exceptions to this pure solvent heuristic.

Perhaps a better approach is to use only pure solvents initially in the screening procedure to determine the best 3 to 5 pure solvents for maximizing yield and, if applicable, the worst 3 to 5 pure solvents for potential counter-recrystallization.  The pairs of binary solvent mixtures formed here with those selected solvents (up to 10 mixtures each, if counter-recrystallization is possible) would then be examined for improved optimal yields.  This strategy would easily allow all 60 or so of the most commonly used solvents in industry to always be directly studied as target solvent candidates.  Using this procedure in the current study would have uncovered 3 of the 4 optimal solvent mixtures for counter-recrystallization listed in Table A.6 but unfortunately none of the 3 optimal solvent mixtures listed in Table A.7.  (Regardless, none of those three solvent mixtures generate maximum yields larger than the top four optimal pure solvents).  The optimal mixtures identified could also perhaps give insight into suggesting which “cousin” solvents in a given class of solvents should also be examined.  (In the current study, Table A.6 indicates that other alcohol species not targeted yet could possibly show promise as alternative solvent mixtures).  Performing this suggested procedure would keep the required search O(N) computationally complex.  Eliminating the consideration of the various possible combinations of solvent mixtures from the three step optimization algorithm (i.e., effectively eliminating Step 2) reduces the number of calculations required to perform each global solvent optimization trial tremendously.  It also reduces the likelihood that any potentially immiscible solvent/solute mixtures will be recommended by the optimization algorithm.

6.0	Conclusions

The overriding goal of this study was to develop a computational means to more readily screen all potential target solvents and operating conditions to determine those candidates which maximize recovery yields of the pure desired compounds via recrystallization separations.  Its objective thus is to identify a handful of the most promising solvent systems to be among those examined further in an experimental investigation.  It attempts to accomplish this with a minimum of experimental data and without any data regression.  The three step optimization algorithm logic described in this study is completely general and therefore can be used in such searches regardless of the specific activity coefficient model employed.  Its overall applicability has been successfully demonstrated in this treatment.

The calculation method for determining mutual saturated solubilities described here has been designed to help systematically predict the optimal operating conditions for recrystallization separations.  Applications of this method using the UNIFAC Equation to estimate Caffeine and Theophylline saturations has given encouraging, although not totally satisfying, results.  However, again, this method is not meant to replace actual experimental measurements.  Rather, the method is meant to be used to indicate which solvents and operating conditions deserve further experimental study.  It has been demonstrated to be good indicators of mutual saturated solubility trends in a wide range of possible saturated solution conditions.  Therefore, it should provide at least moderately useful solubility predictions for screening possible solvent candidates for optimizing recrystallization purifications.

It should be noted that the accuracy of the required mutual saturated solubility predictions improves dramatically when even limited actual solitary saturated solubility data are available.  Thus, as stated by Tanveer et al. [26], experimental data should always be given first priority as the basis for thermodynamic modeling.  As a fallback, predictive models, such as those using the UNIFAC equation, can prove to be at least moderately useful by default when no experimental data are available.  Consequently, the results of models developed with experimental data, however limited those data are, should always be preferred over predictive models.

Even with its present shortcomings, the method described for predicting the required mutual saturated solute solubilities can be applied to a wide variety of pharmaceutical compounds and potential solvents.  This method is meant to streamline the target solvent selection process and hopefully make more efficient use of the limited research resources available.  The calculated results are thus meant to augment human judgments based on past experience, literature reports, and typical engineering heuristics.  Hopefully, the proposed solvent optimization algorithm leads to both increased research productivity and ultimately to more efficient separation processes utilizing recrystallization purifications.

Appendix A.   Computational Target Compound Study Results

Table A.1   Physical Properties of Computational Test Solid Compounds

|  | Caffeine | Theophylline |
| --- | --- | --- |
| Measured normal melting point (K) | 509.31, 508.02, 507.73, 509.54, 510.25, 510.26, 510.07, 508.38 | 544.79, 546.810, 544.011, 542.312, 543.713 |
| Average normal melting point (K) | 509.2 | 544.3 |
| Measured heat of fusion (J/mol) | 248003, 198604, 219005, 193806, 183007, 209508 | 1900010, 2820011, 2820012, 3120013 |
| Average heat of fusion (J/mol) | 20865 | 26650 |

| Temperature (K) | Ideal solubility mole fraction |  | Ideal Solvent Selectivity Ratio (S12) |
| --- | --- | --- | --- |
|  | Caffeine | Theophylline |  |
| 273.15 | 0.0141 | 0.0029 | 4.86 |
| 278.15 | 0.0166 | 0.0035 | 4.74 |
| 283.15 | 0.0195 | 0.0044 | 4.43 |
| 288.15 | 0.0227 | 0.0053 | 4.28 |
| 293.15 | 0.0263 | 0.0064 | 4.11 |
| 298.15 | 0.0304 | 0.0072 | 3.95 |
| 303.15 | 0.0349 | 0.0092 | 3.79 |
| 308.15 | 0.0400 | 0.0109 | 3.67 |
| 313.15 | 0.0455 | 0.0129 | 3.53 |
| 318.15 | 0.0517 | 0.0151 | 3.42 |
| 323.15 | 0.0584 | 0.0177 | 3.30 |

Table A.1   References

1.	Bruns, S. et al., Thermochim. Acta, (1984) 72, 31.

2.	Grady, L. T., et al., J. Pharm. Sci., (1973) 62, 459.

3.	Guo, K., et al., Crystal Growth & Design, (2010) 10, 1, 268 - 273.

4.	Dong, J-X, et al., J. of Chem. Thermo., (2007) 39, 1, 108 - 114.

5.	Pinto, S. S., and H. P. Diogo, J. Chem. Thermo., (2006) 38, 12, 1515 - 1522.

6.	Klous, B., et al., J. Pharm. & Biomed. Analysis, (2005) 39, 5, 944 - 950.

7.	Domalski, E. S., and E. D. Hearing, J. Phys. Chem. Ref. Data, (1990) 19, 4, 881.

8.	Weinstein, D. J., et al., Mol. Crystals and Liq. Crystals, (1984) 104, 1 - 2, 95-109.

9.	Bruns, S. et al., Thermochim. Acta, (1984) 72, 31.

10.	Good, D. J., and N. Rodriguez-Hornedo, Crystal Growth & Design, (2009) 9, 5, 2252 - 2264.

11.	Gondova, T., et al., Thermochimica Acta, (1989) 156, 1, 147 - 155.

12.	Suzuki, E., et al., Chem. Pharm. Bull., (1989) 37, 2, 493 - 497.

13.	Fokkens, F. G., et al., Int. J. of Pharm., (1983) 14, 1, 79 - 93.

Table A.2   Chemical Properties of Computational Test Solid Compounds

| Compound | Molecular Weight (g/mole) | Formula | Compound UNIFAC Functional Groups | Molecular Structure |
| --- | --- | --- | --- | --- |
| Caffeine | 194.19 | C8H10N4O2 | 1 C=C, 2CH2CO, 3CH3N, 1ACNH2 |  |
| Theophylline | 180.16 | C7H8N4O2 | 1 C=C,2 CH2CO, 2CH3N, 1CHNH, 1 ACNH2 |  |

Table A.3   Chemical and Physical Properties of Computational Test Solvent Compounds

| Name | Abbreviation | Molecular Weight (g/mol) | Chemical Formula | UNIFAC Functional Groups | Normal Melting Point (K) | Normal Boiling Point (K) |
| --- | --- | --- | --- | --- | --- | --- |
| Anisole | ANI | 108.14 | C7H8O | 5 ACH, 1 AC, 1CH3O | 235.65 | 426.85 |
| Toluene | TOL | 92.13 | C7H8 | 5 ACH, 1 ACH3 | 178.15 | 383.76 |
| Hexane | HEX | 86.17 | C6H14 | 2 CH3,4 CH2 | 178.15 | 342.04 |
| N-N-Dimethylformamide | DMF | 73.09 | C3H7NO | 1 DMF | 212.04 | 422.04 |
| 1-4 Dioxane | DXN | 88.11 | C4H8O2 | 2 CH2O, 2 CH2 | 284.93 | 374.26 |
| Ethanol | ETH | 46.07 | C2H6O | 1 CH3, 1 CH2, 1 OH | 159.04 | 351.65 |
| Cumene | CMN | 120.20 | C9H12 | 2 CH3, 5 ACH, 1 ACCH | 177.15 | 425.55 |
| Ethyl Acetate | EAC | 88.11 | C4H8O2 | 1 CH2O, 1 CH3CO, 1 CH3 | 189.54 | 350.37 |
| 1-Octanol | OCT | 130.23 | C8H18O | 1 CH3, 7 CH2, 1 OH | 258.15 | 468.15 |
| Acetone | ACT | 58.08 | C3H6O | 1 CH3CO, 1 CH3 | 179.26 | 329.26 |
| Methanol | MTH | 32.04 | CH4O | 1 CH3, 1 OH | 175.60 | 337.85 |
| Carbon Tetrachloride | CTC | 153.82 | CCl4 | 1 CCl4 | 250.19 | 349.87 |
| Methyl Butyl Ketone | MBK | 100.16 | C6H12O | 2 CH3, 1 CH2, 1 CH3CO,1 CH | 215.95 | 394.82 |
| Butyl Acetate | BAC | 116.16 | C6H12O2 | 1 CH3, 3 CH2, 1 CH3COO | 195.26 | 399.25 |
| Tetrahydrofuran | THF | 72.11 | C4H8O | 1 THF, 3 CH2 | 164.65 | 339.25 |
| Xylene | XYL | 106.16 | C8H10 | 4 ACH, 2 ACCH3 | 286.35 | 411.55 |
| Amyl Acetate | AMA | 130.19 | C7H14O2 | 1 CH3, 4 CH2, 1 CH3COO | 202.15 | 422.15 |
| Cyclohexane | CHX | 84.16 | C6H12 | 6 CH2 | 279.65 | 353.87 |
| Aniline | ANL | 93.13 | C6H5NH2 | 5 ACH, 1 ACNH2 | 266.85 | 457.25 |
| Acetonitrile | ACN | 41.05 | C2H3N | 1 CH3CN | 228.15 | 354.76 |

Table A.4   Calculated Optimal Caffeine Recrystallization Yields using just Pure Solvents

| Name | Abbreviation | x1eq | x2eq | S12 | Temperature (K) | Yield | Optimum Solvent Loading (mol) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Anisole | ANI | 5.53E-02 | 1.07E-02 | 5.15 | 323.15 | 0.0903 | 13.1 |
| Toluene | TOL | 6.87E-03 | 1.41E-03 | 4.88 | 323.15 | 0.1393 | 105.6 |
| Hexane | HEX | 3.03E-05 | 6.71E-06 | 4.52 | 323.15 | 0.2018 | 22361.9 |
| N-N-Dimethylformamide | DMF | 1.51E-01 | 4.76E-02 | 3.17 | 323.15 | 0.4406 | 2.5 |
| 1-4 Dioxane | DXN | 6.28E-03 | 2.15E-03 | 2.92 | 323.15 | 0.4839 | 69.3 |
| Ethanol | ETH | 1.39E-01 | 2.28E-02 | 6.11 | 323.15 | 0.0000 | 5.1 |
| Cumene | CMN | 2.31E-03 | 4.28E-04 | 5.41 | 323.15 | 0.0463 | 349.4 |
| Ethyl Acetate | EAC | 1.51E-02 | 4.00E-03 | 3.78 | 323.15 | 0.3326 | 36.8 |
| 1-Octanol | OCT | 3.17E-02 | 5.40E-03 | 5.88 | 323.15 | 0.0000 | 25.8 |
| Acetone | ACT | 1.11E-01 | 2.65E-02 | 4.20 | 323.15 | 0.2589 | 4.9 |
| Methanol | MTH | 1.58E-01 | 2.67E-02 | 5.94 | 323.15 | 0.0000 | 4.4 |
| Carbon Tetrachloride | CTC | 9.35E-04 | 1.55E-04 | 6.04 | 323.15 | 0.0000 | 908.6 |
| Methyl Butyl Ketone | MBK | 2.17E-02 | 4.55E-03 | 4.77 | 323.15 | 0.1577 | 32.1 |
| Butyl Acetate | BAC | 4.47E-03 | 1.13E-03 | 3.94 | 323.15 | 0.3048 | 131.6 |
| Xylene | XYL | 2.55E-03 | 6.54E-04 | 3.90 | 323.15 | 0.3120 | 228.6 |
| Amyl Acetate | AMA | 2.94E-03 | 7.38E-04 | 3.99 | 323.15 | 0.2964 | 202.5 |
| Cyclohexane | CHX | 3.69E-05 | 8.52E-06 | 4.33 | 323.15 | 0.2353 | 17611.9 |
| Aniline | ANL | 1.60E-01 | 3.36E-02 | 4.78 | 323.15 | 0.1573 | 3.6 |
| Acetonitrile | ACN | 1.76E-01 | 3.85E-02 | 4.58 | 323.15 | 0.1920 | 3.1 |
| Tetrahydrofuran | THF | 3.07E-03 | 9.20E-04 | 3.34 | 323.15 | 0.4103 | 162.4 |

Table A.5   Measured and Calculated Single Compound Solubilities

Table A.5   References

- Watson, H. E. et al., “The Extraction of Caffeine,” Diocesan Press, Madras, p. 206, (1923).

- Marrero, J. & J. Abildskov, DECHEMA Chemistry Data Series, Vol. XV, pp. 181-182, (2003).

- Bustamante, P. et al., J. of Pharm. Sci., 91 (3), pp. 874-883, (March 2002).

- Shalmashi, Anvar & Golmohammad, F.. (2010). Solubility of caffeine in water, ethyl acetate, ethanol, carbon tetrachloride, methanol, chloroform, dichloromethane, and acetone between 298 and 323 K. Latin American applied research. 40. 283-285.

- 5.Zhong, J., Tang, N., Asadzadeh, B., & Yan, W. (2017). Measurement and Correlation of Solubility of Theobromine, Theophylline, and Caffeine in Water and Organic Solvents at Various Temperatures. Journal of Chemical & Engineering Data, 62(9), 2570–2577. https://doi.org/10.1021/acs.jced.7b00065

Table A.6   Calculated Improved Optimal Caffeine Yields via Counter Recrystallization

| Percentage Recovery Yield Using a Single Recrystallization |  | Calculated Percent Yield Using 1-4 Dioxane at 50 ̊ C | Calculated Percent Yield Using N-N-Dimethylformamide at 50 ̊ C | Calculated Percent Yield Using Tetrahydrofuran at 50 ̊ C |
| --- | --- | --- | --- | --- |
|  |  | 48.4 | 44.1 | 41.0 |
| Percentage Recovery Yield Using Two Recrystallizations |  |  |  |  |
|  | Overall Calculated Percent Yield Using the Below Conditions for the Second Recrystallization Step |  |  |  |
| Initial Recrystallization Step |  |  |  |  |
| Solvent | Temperature( ̊ C) | 1-4 Dioxane at 50 ̊ C | N-N-Dimethylformamide at 50 ̊ C | Tetrahydrofuran at 50 ̊ C |
| Methanol/Carbon Tetrachloride (0.3/0.7) | 0 | 83.5 | 82.2 | 81.2 |
| Ethanol/Carbon Tetrachloride (0.25/0.75) | 0 | 79.7 | 78.0 | 76.8 |
| 1-Octanol/Carbon Tetrachloride (0.25/0.75) | 0 | 77.9 | 76.1 | 74.8 |
| Aniline/Acetonitrile (0.75/0.25) | 0 | 77.4 | 75.5 | 74.2 |
| Ethanol | 0 | 76.4 | 74.5 | 73.1 |
| Methanol | 0 | 76.3 | 74.3 | 72.9 |
| Carbon Tetrachloride | 0 | 74.8 | 72.7 | 71.2 |
| Aniline | 0 | 72.3 | 70.0 | 68.4 |

Table A.7   Comparison of Selected Best Solvent Mixtures versus Associated Best Pure

Solvent Optimal Caffeine Yields

| Solvent Mixture | Solvent Ratio | Temperature (K) | Percent Yield | Pure Solvent | Temperature (K) | Percent Yield |
| --- | --- | --- | --- | --- | --- | --- |
| Ethyl Acetate/Xylene | 0.45/0.55 | 323.15 | 34.1 | Xylene | 323.15 | 31.2 |
| Butyl Acetate/Xylene | 0.30/0.70 | 323.15 | 32.8 | Xylene | 323.15 | 31.2 |
| Amyl Acetate/Xylene | 0.25/0.75 | 323.15 | 32.4 | Xylene | 323.15 | 31.2 |

List of Symbols and Notations

ai*	activity of saturated compound i in a solitary solute solution

ai**	activity of saturated compound i in a mutual saturated solute solution

ΔHiFUS	heat of fusion of compound i at its normal melting point temperature

L	minimum amount of solvent load causing only pure Compound i to solidify

N	fractional amount of original solid mixture solvated with L solvent loaded

R	Ideal Gas Law constant

Sij	Solvent Selectivity factor between Compounds i and j

si	mole fraction of Compound i in original solid mixture

T	absolute temperature

TMi	absolute normal melting point temperature of Compound i

xi*	mole fraction of saturated Compound i in a solitary solute solution

xi**	mole fraction of saturated Compound i in a mutual saturated solute solution

Y	fractional recovery yield possible of Compound i

References

[1]	Wibowo, C., “Developing Crystallization Processes,” Chemical Engineering Progress, March, 2011, pp. 21 – 31.

[2]	Wibowo, C., “Solid – Liquid Equilibrium: The Foundations of Crystallization Process Design,” Chemical Engineering Progress, March, 2014, pp. 37 – 45.

[3]	Sheikholeslamzadeh, E., C. Chen and S. Rohani, “Optimal Solvent Screening for the Crystallization of Pharmaceutical Compounds from Multisolvent Systems,” Ind. Eng. Chem. Res., Vol. 51, Issue 42, 2012, pp. 13,792 – 13,802.

[4]	Abrams, D. S., and J. M. Prausnitz, “Statistical Thermodynamics of Liquid Mixtures: A New Expression for the Excess Gibbs Energy of Partly or Completely Miscible Systems,” AIChE Journal, Vol. 21, No.1, 1975, pp. 116 – 128.

[5]	Sandler, S. I., Chemical and Engineering Thermodynamics, John Wiley & Sons, New York, NY, 1977, pp. 457 – 462.

[6]	Fredenslund, A., R. L. Jones and J. M. Prausnitz, “Group-Contribution Estimation of Activity Coefficients in Non-Ideal Liquid Mixtures.” AIChE Journal, Vol. 21, No. 6. 1975, pp. 1086 – 1098.

[7]	Wilcox, R. F., “Expanding the Boundaries of UNIFAC,” Chemical Engineering, Vol. 90, No.1, 1983, pp. 66 – 68.

[8]	Hansen, H. K., P. Rasmuesen, A. Fredenslund, M. Schiller, and J. Gmehling, “Vapor-Liquid Equilibria by UNIFAC Group Contribution, 5. Revisions and Extensions,” Ind. Eng. Chem. Res., 1991, pp. 2352 – 2355.

[9]	Smith, J. M., H. C. Van Ness, M. M. Abbott, and M. T. Swihart,  Introduction to Chemical Engineering Thermodynamics, The McGraw-Hill Co., Inc, New York, NY, 2017, pp. 730 – 736.

[10]	Poling, B. E., J. M. Prausnitz, and J. P. O’Connell, The Properties of Gases and Liquids, The McGraw-Hill Co., Inc, New York, 2001, pp. 8.73 – 8.111.

[11]	Witting, R., J. Lohmann, and J. Gmehling, “Vapor-Liquid Equilibria by UNIFAC Group Contribution, 6. Revisions and Extensions,” Ind. Eng. Chem. Res., 42, 2003, pp. 183-188.

[12]	Magnussen, T., P. Rasmuesen, A. Fredenslund, “UNIFAC Parameter Table for Prediction of Liquid-Liquid Equlibria,” Ind. Eng. Chem. Process Des. Dev., 20, 1981, pp. 331 - 339.

[13]	Gonzalez, H. E., J. A. Abildskov, R. Gani, P. Rousseaux, and B. LeBertz, “A Method for Prediction of UNIFAC Group Interaction Parameters,” AIChE Journal, Vol. 53, No. 6, 2007, pp. 1620 – 1632.

[14]	Dang, D. and D. P. Tasslos, “Predictions of Enthalpies of Mixing with a UNIFAC Model,” Ind. Eng. Chem. Process Des. Dev., 25, 1986, pp. 22 - 31.

[15]	Gmehling, J., “Potential of Group Contribution Methods for the Prediction of Phase Equilibria and Excess Properties of Complex Mixtures,” Pure Appl. Chem., Vol. 75, No. 7, 2003, pp. 875 – 888.

[16]	Hill, D., and F. C. Justice, “Understand Thermodynamics to Improve Process Simulations,” Chemical Engineering Progress, December, 2011, pp. 20 – 25.

[17]	Gmehling, J. G., T. F. Anderson and J. M. Prausnitz, “Solid-Liquid Equilibria Using UNIFAC,” Industrial Engineering Chemistry Fundamentals, Vol. 17, No. 4, 1978, pp. 269 – 273.

[18]	Acree, Jr., W. E., and J. H. Rytting, “Solubility in Binary Solvent Systems IV.  Prediction of Naphthalene Solubilities Using the UNIFAC Group Contribution Model,” International Journal of Pharmaceutics, Vol. 10, 1982, pp. 231 – 238.

[19]	Acree, Jr., W. E., Thermodynamic Properties of Non-Electrolytic Solutions, Academic Press, Orlando, Florida, 1984, pp. 233 – 236.

[20]	Martin, A., P. L. Wu, A. Beerbower and J. M. Prausnitz, “Extended Hansen Solubility Approach:  Naphthalene in Individual Solvents”, Journal of Pharmaceutical Sciences, Vol. 20, No. 9, 1981, pp. 1260 – 1264.

[21]	Fickelscherer, R. J., Solvent Optimization in the Recrystallization Purification of Pharmaceutical Products, Masters Thesis, Department of Chemical Engineering, University of Delaware, Newark, Delaware, 1985.

[22]	Smalmashi, A., and F. Golmohammed, “Solubility of Caffeine in Water, Ethylacetate, Ethanol, Carbon Tetrachloride, Methanol, Chloroform, Dichloromethane, and Acetone between 298 and 323 K,” Lat. Am. Appl. Res., Vol 4., No. 3., July, 2010, pp. 283 - 285.

[23]	Pinto, S. S., and H. P. Diago, “Thermochemical Study of Two Anhydrous Polymorphs of Caffeine,” J. Chem. Thermodynamics, Vol. 38, 2006, pp. 1515 – 1522.

[24]	Jelvehgari, M., J. Barar, A. Nokhodchi, S. Shadrou, and H. Valizadeh, “Effects of Process Variables on Micrometric Properties and Drug Release of Non-degradable Microparticles,” Advanced Pharmaceutical Bulletin, Vol. 1, No. 1, 2011, pp. 18 – 26.

[25]	Lee, E. H., “A Practical Guide to Pharmaceutical Polymorph Screening and Selection,” Asian Journal of Pharmaceutical Sciences, Vol. 9., 2014, pp. 163 – 175.

[26]	Tanveer, S., Y.F. Hao, and C.-C. Chen, “Introduction of Solid/Liquid Equilibrium Modeling,” Chemical Engineering Progress, September, 2014, pp. 37 – 45.

Nucleation and crystal growth rate both depend upon the degree of super saturation in the mother liquor, which is the difference between the actual higher concentrations of the solutes in that solution compared with their corresponding saturated solubilities [1].

Obtaining the maximum possible recovery of the desired product is normally given the highest priority in recrystallization operations [2].  Other factors, such as very low mutual saturated solubilities of the target compounds in the solvent system being considered, can also rule out those solvent systems from being selected regardless of their theoretical yields.

Since its inception, the UNIFAC Equation has evolved and grown in popularity as more VLE functional group interaction parameters have been determined [8-11], Liquid-Liquid Equilibrium has been directly handled [12], methods have been developed to predict missing VLE interaction parameters [13] and methods have been developed for direct predictions of Enthalpies of Mixing [14]. Estimating Enthalpies of Mixing and predicting activity coefficients at infinite dilution or for asymmetric systems are still weaknesses of the UNIFAC Equation [15].  Regardless, even in its present form, the UNIFAC Equation represents both a straightforward and generally very flexible method for calculating solution activity coefficients and typically gives good predictive results for a wide variety of applications.  Its continual improvement and expanding versatility has boded well for this equation’s prominence as the most popular group contribution methodology and its general usefulness as the most commonly used predictive activity coefficient model [16]. Its greatest flexibility is that it can generate from either approximate to highly accurate estimates of activity coefficients for a wide range of solutions for which experimental data are not available.  It also does not require any further data regression in order to calculate these activity coefficients.

Caffeine belongs to a group of stimulants called xanthines; its estimated worldwide daily consumption is 250 mg per capita [23].

The following comments on polymorphs of pharmaceutical compounds were summarized from both Lee [25] and Pinto & Diogo [23]:

“Almost every substance can exist in two or more solid phases provided the experimental conditions are suitable (Ostwald 1899).”  Fifty percent of active ingredients in pharmaceutical products show at least two polymorphic forms.  Polymorphs can be obtained when any of the following operating conditions are externally manipulated: 1) the heating and cooling rates; 2) recrystallization temperature; 3) evaporation rates; 4) the degree of super saturation; 5) the rate of agitation; and 6) the solution pH.  Some of the physical thermodynamic properties of the solids of interest that can be altered by having different forms include normal melting point, enthalpy, entropy, saturated solubility, and heat capacity.  Polymorphic pairs are enantiotropically related if the low melting form has a higher heat of fusion; otherwise they are monotropically related.  If the higher melting point form has a higher solubility above the transition temperature, polymorphs are enantiotropically related.  In a monotropic system, the higher melting point form always has a lower solubility.  Polymorphs of this study’s target pharmaceutical products are strongly suspected to be the reason the values of the heats of fusion and normal melting point temperatures reported in the open literature vary so much.  To compensate for this situation, average values of these available physical property measurements were used throughout this study.

Some liberties were taken when defining the UNIFAC functional groups for describing these pharmaceutical molecules.  Specifically, CH2-C=O was used instead of C=O and C(aromatic)-NH2 was used instead of  C(aromatic)-N to describe the test molecules [26].

DMSO and water, the most commonly used solvents in pharmaceutical recrystallizations, were ruled out as potential target solvents for this study because some of the necessary UNIFAC group interaction parameters do not yet exist for all targeted solute/solvent combinations examined in this study.

From Table A.4, the ideal pure solvent Solubility Selectivity factors all range between 3.30 and 4.86 over the 50 K temperature range of this study.

The only 3 exceptions to this generalization encountered in this study are all reported in Table A.7; none of these solvent mixtures would potentially improve optimal yield by more than 3% above that possible with one those solvent mixtures’ pure components.  Additionally, these 3 cases did not affect the choice of the preferred solvent system from this global solvent optimization study; many other possible solvent systems gave better maximum yields.

This is not always an absolutely true generalization:  the four potential best counter recrystallization solvent systems listed in Table A.5 are actually binary solvent mixtures and could potentially improve overall Caffeine yield by over 7 percent above the best pure solvent system for counter-recrystallization.

? It reduces the potential computational complexity of the optimization algorithm from being O(N!) (all possible solvent combinations) to being O(N), where N is the number of unique pure solvents chosen as the potential candidates being considered in the given optimization study  (It should be noted this study only looked at O(N2) possible solvent combinations (i.e., only all 190 possible binary solvent mixtures)).   In this study, ignoring solvent mixtures would have decreased the number of required potential trial equilibrium calculations in the optimization from 43,890 to 220 (i.e., 20 pure solvents each at 11 different possible temperatures).

In our original study [21], LLE was tested for directly for all recommended solvent/solute mixtures.  These calculations proved to sometimes have problems converging, and even when they did, performing the LLE test typically increased the program’s execution time by a factor of 5.